B.Sc. DEGREE EXAMINATION, NOVEMBER 2015.

III YEAR — VI SEMESTER

Elective III — MICROPROCESSOR FUNDAMENTALS

Time : 3 hours	    Max. Marks : 75

SECTION A — (10 × 2 = 20 marks)

Answer any TEN questions.

- What is the function of ALU?

- Give the action of INTR Pin in 8085.

- What is data bus?

- Write the comment of the mnemonics MOV B,A.

- What is the function of ZNR r and DCx  rp?

- What is register indirect addressing? Give examples.

- Define memory mapping.

- Give the pin out signal function diagram of 2k × 8 RAM.

- What are I/O read machine cycle?

- Write the important control signals of 8255.

- Define interrupt service routine.

- Mention the use of TRAP in 8085.

SECTION B — (5 × 5 = 25 marks)

Answer any FIVE questions.

- Describe the use of various registers in 8085.

- Explain the address data and control bus structure of 8085.

- Write the assembly language program for 8 bit multiplication.

- Discuss branching and machine control instruction with example.

- Draw and explain the timing diagram for MVI A instruction.

- Write the output interface programme for flashing LED’s.

- Explain SIM instruction.

SECTION C — (3 × 10 = 30 marks)

Answer any THREE questions.

- Draw the architecture of 8085 microprocessor and detail the pin configuration.

- Discuss the various addressing modes of 8085 with example.

- Explain in detail the function of 2K × 8 ROM interface.

- Draw the internal block diagram and discuss the working of 8255 programmble pheripheral interface.

- With diagram explain the interrupt acknowledge machine cycle.

———————