B.Sc. DEGREE EXAMINATION, NOVEMBER 2015.

III YEAR — V SEMESTER

Major Paper XII — MICROPROCESSOR ARCHITECTURE & PROGRAMMING

Time : 3 hours	    Max. marks : 60

SECTION A — (10 × 1 = 10 marks)

Answer any TEN questions.

- Convert (10011)2 to its equivalent decimal number.

- What is the function of an input device?

- Explain the use of SID pin.

- What are flags?

- What is data transfer instruction?

- How is the instruction set classified?

- Explain the following instructions:

- SHLD 2350H

- STAX B.

- What is direct addressing mode?

- Write an algorithm to perform 8-bit binary addition.

- What do you mean by machine language?

- What is the machine control operations used in 8085 microprocessor?

- Differentiate between static and dynamic RAM.

SECTION B — (5 × 4 = 20 marks)

Answer any FIVE questions.

- Convert the following:

(a)	(3FC.8)H into decimal

(b)	(65.534)10 into hexadecimal.

- Explain the structure of flag register of 8085.

- Explain the various logical instructions of 8085.

- Discuss the different types of JUMP instructions in 8085.

- Write an assembly language program to subtract two 8-bit binary numbers.

- What are RAM’s and ROM’s? Why should both of these be used in an 8085 system?

- Explain how the AD0-AD7 bus is demultiplexed in 8085.

SECTION C — (3 × 10 = 30 marks)

Answer any THREE questions.

- What is a bus? Explain the different buses in a microcomputer system.

- Explain the architecture of 8085 with the help of its internal block schematic.

- Explain the instruction classification.

- What are the various addressing modes available in 8085? Explain with examples.

- Write an assembly language program to arrange the numbers in ascending order.

———————